# SEAT Deep Research v1

UDS / ECU / Sensor Discovery Hypothesis Pack
Research date: 2026-08-30  |  Baseline: uds-map v9

Status: Research hypotheses - additive only - physical validation required before promotion

# 1. Executive summary

SEAT should not be implemented as one universal diagnostic profile. The online evidence supports multiple diagnostic generations: legacy K-line, PQ-era CAN, the unusually mixed Ibiza 6J, MQB, MQB A0, MQB A+ LWB, MQB Evo/SFD, and the Mii Electric shared e-Up architecture.

For modern SEAT vehicles, the strongest strategy is platform classification -> source-backed ECU route probes -> identity fingerprint -> ECU-family matching -> route-specific DIDs. A full blind scan should be a late fallback, not the first action.

The Mii Electric branch is unusually strong: OVMS publishes exact request/response IDs and a large DID set. It also demonstrates why routes cannot be flattened: Mii Electric steering uses 0x712->0x71A, while the broader VAG ODIS-derived catalogue uses 0x712->0x77C for steering assist.

MQB Evo Leon KL vehicles introduce GW2020/SFD. Ross-Tech states SFD protects advanced changes but does not block basic identification, DTC reads or measuring values, so read-only discovery should continue without automatic SFD unlocking.

Ibiza 6J is a critical transport edge case: one documented vehicle uses K-line KWP2000, TP2.0, TP1.6 and UDS across different ECUs. A silent UDS request therefore does not necessarily mean an ECU is absent.

# 2. What success looks like for SEAT

Identify SEAT from VIN/WMI and classify the likely platform branch.

Detect the diagnostic transport without assuming UDS is universal.

Inventory reachable ECUs using known routes before broad enumeration.

Fingerprint each ECU using part/HW/SW/system identity and, where available, ASAM/ODX family.

Apply only compatible model/platform/ECU-family DIDs.

Keep responsive-but-unknown DIDs as valid discoveries.

Record negative responses and terminal states as evidence.

Promote knowledge only after project physical validation; family promotion requires cross-vehicle reproduction.

# 3. Platform and transport segmentation

| Branch | Models/examples | Architecture | Runtime treatment | Confidence |
| --- | --- | --- | --- | --- |
| legacy_kline | Ibiza/Cordoba 6L, Alhambra 7V, other pre-CAN holdovers | K-line ISO9141/KWP1281/KWP2000, sometimes partial CAN for emissions/selected ECUs | kline_if_supported; standard_obd; do_not_assume_uds | high |
| pq35_can | Leon 1P, Altea 5P, Toledo 5P | CAN diagnostics with older VAG transport/protocol mixture; UDS appears on some ECUs | can11_500; tp2_or_uds_by_ecu; platform_route_catalog | high |
| pq25_mixed_ibiza_6j | Ibiza 6J/6P | Mixed K-line KWP2000, CAN TP2.0, CAN TP1.6 and UDS in same vehicle | kline; tp2_0; tp1_6; uds_can11 | high |
| alhambra_7n | Alhambra 7N | VW Sharan-related 7N CAN architecture; separate from MQB | can11_500; platform_specific_routes; uds_or_legacy_can_by_ecu | medium_high |
| mii_electric_shared_up | Mii electric | 11-bit ISO-TP/UDS; shared VW e-Up / Skoda Citigo-e implementation | can11_isotp; exact_route_allowlist; state_aware_polling | high |
| mqb | Leon 5F, Ateca | MQB; UDS/ODX/ASAM over CAN with J533 gateway | can11_500; known_uds_routes_first; identity_fingerprint; bounded_11bit_fallback | high |
| mqb_a0 | Ibiza KJ (Mk5), Arona KJ | MQB A0; UDS/ODX/ASAM over CAN | can11_500; known_vag_routes_first; identity_fingerprint | high |
| mqb_a_lwb | Tarraco KN | MQB A+ / MQB-A LWB; UDS/ODX/ASAM over CAN | can11_500; mqb_route_allowlist; identity_fingerprint | high |
| mqb_evo | Leon KL (Mk4), Leon e-HYBRID | MQB Evo; GW2020; SFD on protected functions; UDS/ASAM basic reads remain available | can11_500; gateway_710_77a_first; sfd_read_only; identity_fingerprint | high |
| special_unresolved | Exeo 3R, Toledo NH, Mii ICE | Known to be VAG-derived but not normalized in this pass | vin_model_identification; passive_first; known_route_only; fallback_with_terminal_state | low |

# 4. Connection playbook

## 1. Identify Brand And Model

Read VIN by standard OBD/UDS paths where available.

If WMI=VSS, select SEAT overlay.

Resolve model/chassis code and approximate production generation before choosing transport branch.

## 2. Passive Transport Detection

Listen before transmitting.

Record standard vs extended CAN IDs and bus activity.

If legacy/6J branch is possible, preserve K-line/TP1.6/TP2.0 as alternate transports rather than assuming UDS.

{"passive_listen_ms": 2000, "can11_expected_bitrate": "500 kbit/s on documented modern VAG diagnostic CAN", "29bit_active_authorized": false}

## 3. Platform Branch

model=Leon KL or gateway part prefix 5WA / GW2020 High -> seat_mqb_evo

model=Leon 5F/Ateca and gateway/ECU prefixes 5Q0 or 3Q0 -> seat_mqb

model=Ibiza KJ/Arona KJ; 2Q0/3Q0 architecture -> seat_mqb_a0

model=Tarraco KN -> seat_mqb_a_lwb

model=Mii electric -> seat_mii_electric_shared_up

model=Ibiza 6J/6P -> seat_pq25_mixed_ibiza_6j

model=Leon 1P/Altea 5P/Toledo 5P -> seat_pq35_can

model=Alhambra 7N -> seat_alhambra_7n

## 4. Known Route Presence And Identity

0x710->0x77A gateway

0x7E0->0x7E8 engine

0x713->0x77D ABS/ESC

0x714->0x77E cluster

0x712->0x77C steering assist (general VAG MQB candidate; NOT Mii electric)

0x70E->0x778 central electronics

0x715->0x77F airbag

0x746->0x7B0 HVAC

optional modules based on platform inventory and VAG route catalogue

Preferred VAG firmware identity request: 0x22 with F187 + F195 + F182; fall back to individual identity DIDs if rejected.

## 5. Fingerprint And Family Match

Store route, supplier, part number, HW/SW, system name and observed ASAM/ODX identity when available.

Match against ECU-family hypotheses before importing sensor decoders.

Part/software compatibility outranks model-name similarity.

## 6. Known Did Queries

Query route-specific source-confirmed model DIDs first.

Then query platform/family candidates.

Then query make-level candidates, preserving ambiguous decoder variants.

Do not enter extended diagnostic session automatically in product mode.

## 7. Bounded Fallback

Try remaining VAG UDS route catalogue entries first.

Only then permit conservative 11-bit enumeration 0x700-0x7F6 excluding 0x7DF.

29-bit enumeration remains disabled unless exact new evidence authorizes a normalized pattern.

SEAT is known but platform remains unresolved

CAN11/UDS transport has been positively detected

known VAG route allowlist did not inventory enough ECUs

## 8. Record Terminal State

unsupported_transport_legacy_kline

unsupported_transport_tp1_6

unsupported_transport_tp2_0

transport_detected_no_diagnostic_response

brand_identified_platform_unknown

gateway_blocked_or_unreachable

vehicle_asleep_or_conditions_not_correct

protected_sfd_write_function

ecu_responsive_identity_unreadable

ecu_identified_no_known_signals

route_responding

signals_observed

candidate_decode

vehicle_confirmed

family_confirmed

aborted_bus_health

aborted_budget

aborted_user

# 5. Addressing and route hypotheses

The VAG ODIS-derived catalogue is used as a bounded presence allowlist, not as proof that every listed ECU exists on every SEAT. The first probes on a modern CAN SEAT should generally be gateway, engine, ABS, cluster, steering, central electronics and airbag.

| Role | Request | Response | Scope |
| --- | --- | --- | --- |
| engine_1 | 0x7E0 | 0x7E8 | VAG candidate / platform-gated |
| transmission | 0x7E1 | 0x7E9 | VAG candidate / platform-gated |
| brakes_abs_esc | 0x713 | 0x77D | VAG candidate / platform-gated |
| adaptive_cruise_1 | 0x757 | 0x7C1 | VAG candidate / platform-gated |
| kessy_access_start | 0x732 | 0x79C | VAG candidate / platform-gated |
| airbag | 0x715 | 0x77F | VAG candidate / platform-gated |
| instrument_cluster | 0x714 | 0x77E | VAG candidate / platform-gated |
| hvac | 0x746 | 0x7B0 | VAG candidate / platform-gated |
| central_electronics | 0x70E | 0x778 | VAG candidate / platform-gated |
| gateway | 0x710 | 0x77A | VAG candidate / platform-gated |
| drive_motor | 0x7E6 | 0x7EE | VAG candidate / platform-gated |
| steering_assist | 0x712 | 0x77C | VAG candidate / platform-gated |
| tpms | 0x70B | 0x775 | VAG candidate / platform-gated |
| information_electronics_1 | 0x773 | 0x7DD | VAG candidate / platform-gated |
| battery_charger_charge_management | 0x765 | 0x7CF | VAG candidate / platform-gated |
| telematics | 0x767 | 0x7D1 | VAG candidate / platform-gated |
| hv_battery_energy_control | 0x7E5 | 0x7ED | VAG candidate / platform-gated |
| front_driver_assist_sensor | 0x74F | 0x7B9 | VAG candidate / platform-gated |

# 6. Mii Electric exact route exception set

These are exact routes from the operational OVMS shared e-Up/Mii Electric/Citigo-e implementation [S06], not generic SEAT routes.

| ECU role | Request | Response |
| --- | --- | --- |
| motor_electronics | 0x7E0 | 0x7E8 |
| brakes | 0x713 | 0x77D |
| instrument_cluster_mfd | 0x714 | 0x77E |
| brake_booster | 0x73B | 0x743 |
| steering_assist | 0x712 | 0x71A |
| electric_drive | 0x7E6 | 0x7EE |
| information_electronics | 0x773 | 0x7DD |
| immobilizer | 0x711 | 0x77B |
| hv_battery_management | 0x7E5 | 0x7ED |
| brake_system_sensors | 0x762 | 0x76A |
| hv_charger | 0x744 | 0x7AE |
| hv_charge_management | 0x765 | 0x7CF |

# 7. ECU fingerprint and family strategy

Identity should be captured before proprietary DID reuse. openDBC's VAG firmware query uses manufacturer spare part number F187, manufacturer ECU software version F195 and application-data identification F182 in a multi-DID ReadDataByIdentifier request. When unsupported, fall back to individual standard identity DIDs such as F18A/F18C/F190/F191/F197.

## vag_gateway_mqb_conti_nf - gateway

Identifiers: EV_GatewConti, EV_GatewNF, GW MQB High

Part prefixes observed: 5Q0, 3Q0

Reuse rule: Reuse route/identity behavior first. Promote DIDs only after matching ECU part/software or ASAM family.

## vag_gateway_mqb2020 - gateway

Identifiers: GW2020 High, EV_GatewMQB2020

Part prefixes observed: 5WA

Reuse rule: Strong MQB Evo classifier; SFD may be present, but basic reads remain permitted.

## continental_mk100_ipb - brakes_abs_esc

Identifiers: EV_Brake1UDSContiMK100IPB

Part prefixes observed: 5Q0 614 517

Reuse rule: Excellent candidate for cross-SEAT/VAG family reuse; decodes require software-family compatibility.

## continental_mk100_esp - brakes_abs_esc

Identifiers: EV_Brake1UDSContiMK100ESP

Reuse rule: Keep separate from MK100IPB until route/DID compatibility is physically compared.

## mqb_eps - steering_assist

Identifiers: EV_SteerAssisMQB, EPS_MQB_ZFLS

Reuse rule: Route 712->77C is broad VAG candidate; Mii electric is a known exception at 712->71A.

## mqb_kessy_hella - access_start_authorization

Identifiers: VWKESSYMQB, EV_KessyHellaMQBAB

Reuse rule: Use ASAM identity to bind before importing any DIDs.

## mqb_evo_brake_booster_bosch - brake_booster

Identifiers: EV_BrakeBoostG2BoschVW37W

Reuse rule: MQB Evo specific candidate; source scan shows SFD flag.

# 8. DID research findings

The structured JSON contains 251 DID records. They are intentionally separated by model/platform/route. The make-level OBDb source is useful as a hypothesis generator but contains mixed platforms and competing decode variants.

| Module role | DID records |
| --- | --- |
| hv_charger | 41 |
| hvac | 27 |
| engine | 23 |
| instrument_cluster | 22 |
| gateway | 19 |
| brakes_abs_esc | 18 |
| motor_electronics | 18 |
| hv_battery_management | 18 |
| awd_control | 13 |
| electric_drive | 11 |
| hv_charge_management | 8 |
| airbag | 5 |
| instrument_cluster_mfd | 5 |
| tpms | 4 |
| thermal_management_unknown | 3 |
| transmission_candidate | 3 |
| steering_assist | 2 |
| telematics | 2 |
| brake_booster | 2 |
| information_electronics | 2 |
| transmission | 1 |
| central_electronics | 1 |
| differential_lock_electronics | 1 |
| brakes | 1 |
| brake_system_sensors | 1 |

## 9. High-value model-specific signals

0x714->0x77E  0x1014  ambient_air_temperature | decode: {"encoding": "u8", "scale": 0.5, "bias": -50, "unit": "degC"}

0x714->0x77E  0x202F  engine_oil_temperature | decode: {"encoding": "u8", "scale": 1.0, "bias": -58, "unit": "degC"}

0x714->0x77E  0x2203  odometer | decode: {"encoding": "u24_be", "scale": 1.0, "unit": "km"}

0x714->0x77E  0x2206  fuel_level_raw_or_liters | decode: {"encoding": "u8", "unit": "source_labels_liters_scale_unresolved"}

0x714->0x77E  0x2211  charge_air_pressure_source_quantity | decode: {"encoding": "u8", "scale": "100/255", "unit": "source_percent"}

0x714->0x77E  0x224B  oil_thermal_wear_distance | decode: {"encoding": "u16_be", "scale": 1.0, "unit": "km"}

0x714->0x77E  0x2294  distance_to_empty | decode: {"encoding": "u16_be", "scale": 1.0, "unit": "km"}

0x714->0x77E  0x2299  average_fuel_consumption_source_quantity | decode: {"encoding": "u16_be", "scale": 0.1, "unit": "source_labels_liters"}

0x714->0x77E  0x229A  fuel_level_input | decode: {"encoding": "u16_be", "scale": 0.125, "unit": "liters"}

0x7E1->0x7E9  0x2104  automatic_transmission_fluid_temperature | decode: {"encoding": "u8", "scale": 1.0, "bias": -40, "unit": "degC"}

0x714->0x77E  0x1014  ambient_air_temperature | decode: {"encoding": "u8", "scale": 0.5, "bias": -50, "unit": "degC"}

0x714->0x77E  0x202F  engine_oil_temperature | decode: {"encoding": "u8", "bias": "source_variant", "unit": "degC"}

0x714->0x77E  0x2203  odometer | decode: {"encoding": "u24_be", "scale": 1.0, "unit": "km"}

## 10. Mii Electric battery data

0x1E3B: hv_battery_voltage | raw / 4.0 V

0x1E3D: hv_battery_current | ((raw - 2044.0) / 4.0) * -1 A

0x028C: absolute_soc | raw / 2.5 percent

0x02BD: odometer

0x1E32: energy_counters

0x1E33: max_cell_voltage

0x1E34: min_cell_voltage

0x2A0B: battery_temperature

0x1E0E: max_battery_temperature

0x1E0F: min_battery_temperature

0x1E40-0x1EA5: individual_cell_voltages | raw / 4096 V

0x1EAE-0x1EBD: individual_cell_temperatures | raw / 64 degC

0x7425: cell_temperature_module_17_gen1

0x74CF: history_total_ac_ccs_regen_ah

0x74CB: soh_capacity_ah_cells

0x74CC: cell_ah_history

0x74BE: standing_times_full_empty

0x74D0: total_standing_and_charging_time

# 11. Diagnostic sessions, SFD and safety

Product mode stays in the default session and automatically denies SecurityAccess, writes, routines, resets, DTC clearing and flashing. Non-default session 0x03 is a research-mode feature only when an exact route/DID source requires it and the ECU fingerprint matches.

For Mii Electric, OVMS explicitly starts extended diagnostic session 0x03 on the brake ECU before reading TPMS DID 0x1821 [S06/S07]. That is platform-specific evidence; it is not permission to enter 0x03 on arbitrary SEAT modules.

Ross-Tech states SFD does not interfere with basic controller identification, DTC reads or measuring values [S11]. Therefore an MQB Evo SEAT should remain discoverable in read-only mode without SFD unlocking.

# 12. Request budgets and timeouts

## Known Route Presence

{"max_requests": 64, "rate_requests_per_second": 4, "burst_max": 4, "max_outstanding": 1, "timeout_retry": 0}

## Identity Per Ecu

{"max_requests": 7, "rate_requests_per_second": 2, "max_outstanding": 1, "timeout_retry": 1, "retry_backoff_ms": 250}

## Remaining Vag Route Catalog

{"max_requests": 96, "rate_requests_per_second": 4, "max_outstanding": 1, "timeout_retry": 0}

## Generic Can11 If Authorized

{"from": "0x700", "to": "0x7F6", "exclude": ["0x7DF"], "max_requests": 247, "rate_requests_per_second": 4, "max_outstanding": 1, "timeout_retry": 0}

## Global

{"max_requests": 600, "max_wallclock_ms": 180000, "phase_cooldown_ms": 500}

## Timeouts / aborts

{
  "presence_first_response_ms": 250,
  "identity_first_response_ms": 500,
  "isotp_multiframe_completion_ms": 3000,
  "response_pending_max_ms": 3000,
  "abort_immediately_on": [
    "CAN bus-off",
    "user_cancel",
    "adapter_reports_unsafe_bus_state"
  ],
  "abort_phase_after": {
    "transport_protocol_errors": 8,
    "malformed_isotp_responses": 5,
    "bus_error_escalations": 3
  },
  "no_response_policy": "Silence is normal for absent routes; finish bounded allowlist/range unless budget is hit. On legacy/mixed SEAT platforms, silence may also indicate wrong transport."
}

# 13. Negative-response behavior

| NRC | Meaning | Action | Notes |
| --- | --- | --- | --- |
| 0x10 | generalReject | stop_current_request | {} |
| 0x11 | serviceNotSupported | mark_service_unsupported | {} |
| 0x12 | subFunctionNotSupported | mark_subfunction_unsupported | {} |
| 0x13 | incorrectMessageLengthOrInvalidFormat | fallback_multi_did_to_single_did_or_stop_format | {} |
| 0x21 | busyRepeatRequest | retry_with_backoff | {'max_retries': 2, 'backoff_ms': 500} |
| 0x22 | conditionsNotCorrect | terminal_current_state | {} |
| 0x25 | noResponseFromSubnetComponent | gateway_or_subnet_unreachable | {} |
| 0x31 | requestOutOfRange | mark_did_absent_on_this_ecu | {} |
| 0x33 | securityAccessDenied | protected_terminal_no_security_attempt | {} |
| 0x35 | invalidKey | protected_terminal_no_security_attempt | {} |
| 0x36 | exceedNumberOfAttempts | protected_terminal_no_security_attempt | {} |
| 0x37 | requiredTimeDelayNotExpired | protected_terminal_no_security_attempt | {} |
| 0x78 | requestCorrectlyReceivedResponsePending | wait_without_resend | {'max_wait_ms': 3000} |
| 0x7E | subFunctionNotSupportedInActiveSession | record_session_mismatch_do_not_switch_generically | {} |
| 0x7F | serviceNotSupportedInActiveSession | record_session_mismatch_do_not_switch_generically | {} |

# 14. Representative physical module inventories

These are examples from real VCDS AutoScans. They are observations, not mandatory module lists.

## Leon 5F 2013 - 5F-SE37 (5Q0)

Addresses: 01, 02, 03, 08, 09, 10, 13, 15, 16, 17, 19, 42, 44, 52, 55, 5F, A5  | Sources: S18

## Leon 5F 2020 - 5F-SE37 (3Q0)

Addresses: 01, 03, 08, 09, 10, 13, 15, 17, 19, 42, 44, 4B, 52, 5F, BB, BC, D6, D7  | Sources: S19

## Leon KL eTSI - KL-SE38 (5WA)

Addresses: 01, 02, 03, 05, 08, 09, 10, 13, 15, 17, 19, 23, 2B, 3C, 42, 44, 52, 5F, 61, 6C, 6D, 75, A5, BB, BC, C4, CC, D6, D7, 811E, 811F, 8120, 8121, C002, C003  | Sources: S20

## Ibiza KJ 2019 - KJ-SE27 (2Q0)

Addresses: 01, 03, 05, 08, 09, 10, 15, 17, 19, 2B, 44, 55, 5F, CA, D6, D7  | Sources: S22

## Arona KJ - KJ-SE27 (3Q0)

Addresses: 01, 02, 03, 05, 08, 09, 13, 15, 16, 17, 19, 2B, 44, 5F, 6C  | Sources: S23

## Tarraco KN 2019 - KN-SE37 (3Q0)

Addresses: 01, 02, 03, 05, 08, 09, 10, 13, 14, 15, 17, 18, 19, 22, 2B, 36, 3C, 42, 44, 4B, 52, 5F, 6C, 6D, 75, A5, D6, D7  | Sources: S24

# 15. Conflicts that must remain explicit

## seat_eps_route_712

Finding: General VAG route catalogue maps steering assist 0x712->0x77C, while the Mii electric OVMS implementation uses 0x712->0x71A.

Resolution: Keep both. 0x712->0x71A is exact Mii-electric platform evidence; 0x712->0x77C is a broader VAG/MQB candidate.

## seat_obdb_model_empty_vs_make_data

Finding: Current model defaults for Ateca, Arona, Tarraco, Alhambra, Altea and Mii Electric are empty while the SEAT make-level fallback and other sources contain useful data.

Resolution: Empty model repository state is not negative vehicle evidence and must never delete local or make-level knowledge.

## seat_make_level_mixed_decoders

Finding: SEAT make-level OBDb fallback mixes PQ/MQB/electrified commands and contains competing interpretations/formulas for some DIDs.

Resolution: Store decoder variants keyed by route+platform+ECU fingerprint; no brand-global decoder promotion.

## seat_29bit_fc00_unresolved

Finding: Make-level OBDb contains commands with hdr=FC00, rax=FE0077, pri=17, tst=F1 for ATF-temperature variants.

Resolution: Treat as transport-normalization research only. It does not authorize generic 29-bit enumeration.

## seat_legacy_silence

Finding: Ibiza 6J can mix K-line KWP2000, TP2.0, TP1.6 and UDS across ECUs.

Resolution: UDS silence can mean wrong transport rather than ECU absence.

# 16. Validation plan

Priority physical fleet:

Leon 5F: Large Spanish parc; core MQB and cross-VAG reuse

Ibiza KJ: Large Spanish parc; MQB A0

Leon KL: MQB Evo + SFD/GW2020

Arona KJ: MQB A0 SUV; compare with Ibiza

Ateca: MQB SUV; MK100 IPB family

Mii electric: Strong exact EV route/DID source and known steering-route exception

Ibiza 6J: Mixed transport edge case

Per-vehicle workflow:

Record VIN/model/year/powertrain and adapter/firmware version.

Passive capture before requests.

Execute known-route presence phase within budget.

Read identity DIDs; save raw payloads.

Classify ECU family.

Query only model/platform/family candidates in default session.

Record all positive and negative responses, response times and raw payload lengths.

For candidate physical signals, perform controlled human actions (stationary where possible) and timestamp annotations.

Only use research-mode extended session when exact source+fingerprint gate is satisfied.

Compare the same ECU family on a second vehicle before family promotion.

# 17. Research gaps / next work

P0 - Physical SEAT validation fleet: At least one Leon 5F, Leon KL, Ibiza KJ, Arona, Ateca and Mii electric in Spain; capture route/identity/DID responses with project hardware.

P0 - Normalize OBDb unusual 29-bit-looking commands: Reconstruct exact ELM/OBDb header semantics for FC00/FE0077 pri=17 tst=F1 and capture a real transaction before execution.

P0 - ECU identity fingerprint database: Capture F187/F195/F182 plus F18A/F18C/F191/F197 on responsive modules and bind ASAM/part/software families.

P1 - Ibiza 6J transport support: Decide whether product hardware/software will implement K-line, TP2.0 and TP1.6 or explicitly report unsupported alternate transport.

P1 - Exeo 3R: Dedicated Audi-B7-derived diagnostic research; do not reuse MQB.

P1 - Toledo NH: Resolve platform/transport and model-specific ECU inventory.

P1 - Mii ICE: Separate from Mii electric; obtain exact ECU routes and transport behavior.

P1 - Alhambra 7N: Extract exact Sharan/Alhambra routes and DIDs from current sources.

P2 - DID decoder verification: For each high-value candidate, compare source formula against physical action/reference sensor and at least two compatible vehicles before family promotion.

P2 - SFD/SFD2 future behavior: Continue to verify that read-only identification/measuring values remain accessible on newer SEAT models and document any diagnostic-filter exceptions.

# 18. Source ledger

[S00] Current project uds-map.json
URL: local:/mnt/data/uds-map.json
Type: project_knowledge_map | Licence: project_internal | Retrieved: 2026-08-30
Revision: map-v9
Scope: Current SEAT project knowledge

[S01] OBDb SEAT make-level fallback
URL: https://github.com/OBDb/SEAT
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-30
Revision: default.json blob 3e42a533fb22273ebcc09e03d0769c7bfeeef5a9
Scope: SEAT make-level fallback; mixed models/platforms
Notes: Make-level fallback; do not flatten mixed platform decoders into one universal SEAT truth.

[S02] OBDb Seat-Leon default signalset
URL: https://github.com/OBDb/Seat-Leon/blob/main/signalsets/v3/default.json
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-30
Revision: 86a877653b8090eb0a25b20f53283a8d5cd4b5c7
Scope: SEAT Leon; source year filters apply

[S03] OBDb Seat-Ibiza default signalset
URL: https://github.com/OBDb/Seat-Ibiza/blob/main/signalsets/v3/default.json
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-30
Revision: 7f98a5d6ec28897258644a6909c22b83fd299d09
Scope: SEAT Ibiza; source year filters apply

[S04] VAG UDS CAN IDs extracted from ODIS
URL: https://github.com/ConnorHowell/vag-uds-ids
Type: community_odis_extraction | Licence: NOASSERTION | Retrieved: 2026-08-30
Revision: readme blob 27b5431ed22a10a41095517b88dc95b3ae212441
Scope: VAG group UDS route catalogue; not SEAT-specific
Notes: No licence file found in retrieved repository root; use as reference evidence, not code/data redistribution.

[S05] openDBC Volkswagen platform and firmware-query definitions
URL: https://github.com/commaai/opendbc/blob/master/opendbc/car/volkswagen/values.py
Type: open_source_vehicle_implementation | Licence: MIT | Retrieved: 2026-08-30
Scope: Volkswagen Group platform classification, WMI and firmware query logic

[S06] OVMS VW e-Up / SEAT Mii electric OBD definitions
URL: https://github.com/openvehicles/Open-Vehicle-Monitoring-System-3/blob/master/vehicle/OVMS.V3/components/vehicle_vweup/src/vweup_obd.h
Type: open_source_vehicle_implementation | Licence: MIT | Retrieved: 2026-08-30
Revision: 1a79c553654b4b981c162b6cbb740c9784408d96
Scope: VW e-Up / SEAT Mii electric / Skoda Citigo-e iV shared implementation

[S07] OVMS VW e-Up / SEAT Mii electric OBD decoder
URL: https://github.com/openvehicles/Open-Vehicle-Monitoring-System-3/blob/master/vehicle/OVMS.V3/components/vehicle_vweup/src/vweup_obd.cpp
Type: open_source_vehicle_implementation | Licence: MIT | Retrieved: 2026-08-30
Scope: Mii electric shared platform decoder and polling state logic

[S08] Ross-Tech VCDS CAN-Bus Information
URL: https://www.ross-tech.com/vcds/canbus.php
Type: professional_diagnostic_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: VAG diagnostic transport generations and SEAT chassis examples

[S09] Ross-Tech VCDS Revision History
URL: https://www.ross-tech.com/vcds/revisions.php
Type: professional_diagnostic_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: TP1.6 support required for Ibiza 6J; UDS/ODX/ASAM support history

[S10] Ross-Tech: 2014 SEAT Ibiza 6J protocol discussion
URL: https://forums.ross-tech.com/index.php?threads/2835/
Type: physical_vehicle_autoscan_and_expert_commentary | Licence: reference_only | Retrieved: 2026-08-30
Scope: 2014 Ibiza 6J; protocol used per ECU
Notes: Ross-Tech administrator states K-line KWP2000, TP2.0, TP1.6 and UDS coexist in one car.

[S11] Ross-Tech SFD Wiki
URL: https://wiki.ross-tech.com/wiki/index.php/SFD
Type: professional_diagnostic_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: VAG SFD behavior

[S12] SEAT official: Ibiza MQB A0
URL: https://www.seat-cupra-mediacenter.es/SEAT-Brand/presskits/ibiza/platform-and-measures
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: SEAT Ibiza Mk5

[S13] SEAT official: Arona MQB A0
URL: https://www.seat-cupra-mediacenter.es/SEAT-Brand/presskits/nuevo_arona/Introduccion
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: SEAT Arona

[S14] SEAT official: Leon III MQB
URL: https://www.seat.com/company/news/cars/seat-leon-one-million-solds
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: SEAT Leon III (5F)

[S15] SEAT official: Ateca MQB
URL: https://www.seat-cupra-mediacenter.es/SEAT-Brand/presskits/MotorSEATAteca/carpeta-de-prensa
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: SEAT Ateca

[S16] SEAT official: Tarraco MQB A+
URL: https://www.seat.es/sobre-seat/noticias/coches/seat-tarraco-presentacion-suv
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: SEAT Tarraco

[S17] SEAT official: Leon IV MQB Evo
URL: https://www.seat-mediacenter.com/content/dam/seat-media-center/allnews/press-kits/new-seat-leon-e-hybrid/New-SEAT-Leon-e-HYBRID-the-brands-first-plug-in-hybrid-model-reaches-the-market.pdf
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-30
Scope: SEAT Leon IV / e-HYBRID; MQB Evo

[S18] Ross-Tech: 2013 SEAT Leon 5F MQB AutoScan
URL: https://forums.ross-tech.com/index.php?threads/12734/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: 2013 Leon 5F; 5Q0

[S19] Ross-Tech: 2020 SEAT Leon 5F AutoScan
URL: https://forums.ross-tech.com/index.php?threads/38198/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: 2020 Leon 5F; 3Q0

[S20] Ross-Tech: SEAT Leon KL MQB Evo AutoScan
URL: https://forums.ross-tech.com/index.php?threads/45900/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: Leon KL-SE38; 5WA gateway architecture

[S21] Ross-Tech: Leon KL gateway EV_GatewMQB2020
URL: https://forums.ross-tech.com/index.php?threads/46582/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: Leon KL-SE38; gateway and brake-booster ASAM identities

[S22] Ross-Tech: 2019 Ibiza KJ MQB A0 AutoScan
URL: https://forums.ross-tech.com/index.php?threads/35631/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: Ibiza KJ-SE27; 2Q0

[S23] Ross-Tech: Arona KJ MQB A0 AutoScan
URL: https://forums.ross-tech.com/index.php?threads/32772/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: Arona KJ-SE27; 3Q0; KESSY ASAM identity

[S24] Ross-Tech: 2019 Tarraco KN AutoScan
URL: https://forums.ross-tech.com/index.php?threads/45480/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: Tarraco KN-SE37; 3Q0

[S25] Ross-Tech: Ateca Continental MK100 IPB
URL: https://forums.ross-tech.com/index.php?threads/10927/
Type: physical_vehicle_measurement_map | Licence: reference_only | Retrieved: 2026-08-30
Scope: 2017 Ateca; ABS/ESP Continental MK100 IPB

[S26] Ross-Tech: Leon MK100 IPB family evidence
URL: https://forums.ross-tech.com/index.php?threads/11740/
Type: physical_vehicle_autoscan | Licence: reference_only | Retrieved: 2026-08-30
Scope: 2017 Leon; EV_Brake1UDSContiMK100IPB

[S27] OBDb model repositories currently empty at retrieved revision
URL: https://github.com/OBDb
Type: repository_state | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-30
Revision: common empty default blob 7176ccfbcd8055c9fb74a97f35ef5f197efd1f53
Scope: Seat-Ateca, Seat-Arona, Seat-Tarraco, Seat-Alhambra, Seat-Altea, Seat-Mii-Electric current defaults
Notes: Empty model default is NOT negative evidence about vehicle diagnostic capability.

# 19. Import guidance

Import this pack as a research overlay. Do not replace the existing SEAT object. A suggested merger should compare route+platform+ECU fingerprint, retain conflicts, and require explicit promotion to vehicle_confirmed/family_confirmed. The most immediately actionable additions are platform segmentation, the 6J mixed-transport terminal states, the Mii Electric exact route/DID set, MQB/MQB Evo family fingerprints, and the SFD read-only rule.