# Volkswagen / Audi (VAG) Deep Research v1

UDS / ECU / Sensor Discovery Hypothesis Pack
Research date: 2026-08-31 | Baseline uds-map v9

Status: additive online-research hypotheses; project physical validation required before promotion.

# 1. Executive summary

The current VAG profile must branch by architecture before proprietary discovery. Volkswagen spans PQ, MQB/MQB A0/MQB Evo and MEB; Audi adds longitudinal MLB, adapted MLB evo EV, J1, PPE and PPC.

The strongest common primitive is ECU identity. openDBC uses WMI + chassis code for platform matching and a multi-DID UDS firmware request containing F187, F195 and F182.

MEB evidence is strong: VW ID.4 exposes rich gateway energy-management DIDs on 710->77A and charging DIDs on 744->7AE.

Audi longitudinal evidence proves why negative data matters: the 2015 Q5 supports dense 7E0/7E1 data, while the 2022 RS e-tron GT supports a very different EV/J1 set and explicitly rejects many generic VAG candidates.

PPE/PPC are platform-confirmed but route-knowledge-incomplete in this pass. The correct runtime outcome is a terminal research state, not inherited MQB/MLB assumptions.

# 2. Platform branches

| Branch | Brand | Examples | Architecture | Confidence |
| --- | --- | --- | --- | --- |
| vw_pq | Volkswagen | Golf Mk5/Mk6, Jetta Mk6, Caddy Mk3, Sharan Mk2 | PQ-family; CAN diagnostics with older VAG protocol/UDS mix depending ECU | high |
| vw_mqb | Volkswagen | Golf Mk7, Passat B8, Tiguan Mk2, T-Roc, Arteon | MQB; UDS/ODX/ASAM via J533 gateway | high |
| vw_mqb_a0 | Volkswagen | Polo Mk6, T-Cross | MQB A0; UDS over CAN; some later cars combine gateway+BCM | high |
| vw_mqb_evo | Volkswagen | Golf Mk8, newer MQB Evo derivatives | MQB Evo / GW2020 / SFD-capable | medium_high |
| vw_meb_gen1 | Volkswagen | ID.3, ID.4 2021-23, ID.5 | MEB generation 1 | high |
| vw_meb_gen2 | Volkswagen | ID.4 2024-25 and newer MEB gen2 examples | MEB generation 2; openDBC uses distinct generated DBC flag | high |
| vw_commercial | Volkswagen | Crafter, Transporter/Caravelle/California | Commercial-vehicle branches; MQB/PQ-like families vary by model | medium |
| audi_mqb | Audi | A3 8V, Q2, Q3 F3 | Transverse MQB | high |
| audi_mlb | Audi | Q5 8R, longitudinal Audi generations | MLB longitudinal; route/DID grammar differs materially from MQB/MEB | high |
| audi_mlb_evo_ev | Audi | Audi e-tron SUV / Q8 e-tron first architecture | Adapted MLB evo battery-electric architecture | high |
| audi_meb | Audi | Q4 e-tron | MEB | high |
| audi_j1 | Audi | e-tron GT, RS e-tron GT | J1 performance EV platform | high |
| audi_ppe | Audi | Q6 e-tron, A6 e-tron | PPE + E3 1.2 electronics | high |
| audi_ppc | Audi | new A5, new Q5 family | PPC + E3 electronics | high |

# 3. Runtime connection playbook

## 1. Identify Brand And Platform

Use WMI + VDS/chassis code. openDBC explicitly uses both WMI and chassis code for VW/Audi platform matching.

Classify at minimum PQ/MQB/MEB/MLB/J1/PPE/PPC before proprietary scanning.

## 2. Passive Transport Detection

Listen before transmitting; record 11-bit/29-bit and adapter multiplexing path.

Do not infer 29-bit support from FC00 OBDb syntax until normalized.

## 3. Known Route Inventory

0x710->0x77A gateway

0x7E0->0x7E8 engine

0x7E1->0x7E9 transmission when present

0x713->0x77D brakes

0x714->0x77E cluster

0x712->0x77C steering candidate

0x744->0x7AE charger on electrified platforms

0x7E5->0x7ED HV battery on electrified platforms

## 4. Firmware Identity

Preferred VAG multi-DID request: ReadDataByIdentifier F187 + F195 + F182.

openDBC uses RX offset 0x6A for selected SRS/EPS/radar/camera queries and regular addressing for engine/transmission.

Fallback to individual identity DIDs if multi-DID is rejected.

## 5. Platform Specific Dids

Use exact physical/model support evidence before make-level candidates.

Preserve explicit unsupported-command evidence as platform-scoped negative evidence.

## 6. Bounded Fallback

Run remaining VAG route catalogue before generic 11-bit enumeration.

Generic 11-bit: 0x700-0x7F6 excluding 0x7DF, only when CAN11/UDS is confirmed.

29-bit generic enumeration disabled in this profile.

## 7. Terminal State

platform_known_route_knowledge_incomplete

transport_detected_no_diagnostic_response

known_routes_silent

gateway_blocked_or_unreachable

protected_sfd_write_function

unresolved_transport_encoding

ecu_identified_no_known_signals

route_responding

signals_observed

candidate_decode

vehicle_confirmed

family_confirmed

aborted_budget

# 4. ECU addressing

The ODIS-derived VAG route catalogue is a bounded presence allowlist. It is not a mandatory module list.

| Role | Request | Response |
| --- | --- | --- |
| engine_1 | 0x7E0 | 0x7E8 |
| transmission | 0x7E1 | 0x7E9 |
| brakes_abs_esc | 0x713 | 0x77D |
| adaptive_cruise_1 | 0x757 | 0x7C1 |
| kessy_access_start | 0x732 | 0x79C |
| airbag | 0x715 | 0x77F |
| instrument_cluster | 0x714 | 0x77E |
| hvac | 0x746 | 0x7B0 |
| central_electronics | 0x70E | 0x778 |
| gateway | 0x710 | 0x77A |
| drive_motor | 0x7E6 | 0x7EE |
| steering_assist | 0x712 | 0x77C |
| tpms | 0x70B | 0x775 |
| information_electronics_1 | 0x773 | 0x7DD |
| battery_charger_charge_management | 0x765 | 0x7CF |
| telematics | 0x767 | 0x7D1 |
| hv_battery_energy_control | 0x7E5 | 0x7ED |
| front_driver_assist_sensor | 0x74F | 0x7B9 |

# 5. ECU identity / fingerprinting

Preferred VAG identity probe: UDS ReadDataByIdentifier containing F187 (manufacturer spare part number) + F195 (ECU software version) + F182 (application data identification). If rejected, retry individual identity DIDs within budget.

WMI + chassis code are first-class platform selectors. Part/software compatibility outranks badge/model similarity when reusing ECU-family sensor knowledge.

# 6. DID evidence

The JSON pack contains 217 DID records plus 22 explicit negative-support records. Shared make-level records remain hypotheses; model support matrices outrank them.

## VW ID.4 / MEB high-value examples

0x710->0x77A 0x15D1 electric_motor_power_limit_and_energy

0x710->0x77A 0x2A0E 12v_battery_internal_resistance

0x710->0x77A 0x2A53 dcdc_low_voltage_and_current

0x710->0x77A 0x2AB2 hv_battery_max_energy

0x710->0x77A 0x2AB5 estimated_reserve_range

0x710->0x77A 0x2AB6 displayed_range

0x710->0x77A 0x2AB7 average_consumption_categories

0x710->0x77A 0x2AB8 hv_battery_energy

0x710->0x77A 0x2AF7 12v_battery_extended_metrics

0x744->0x7AE 0x103D onboard_charger_hvdc_current

0x744->0x7AE 0x103E cycle_energy_transformation

0x744->0x7AE 0x1507 charger_rail_energy

0x744->0x7AE 0x1525 socket_a_proximity_voltage

## Audi Q5 2015 longitudinal examples

0x7E0->0x7E8 0x100C fuel_level_2

0x7E0->0x7E8 0x105C generator_load

0x7E0->0x7E8 0x105E battery_voltage_2

0x7E0->0x7E8 0x113D o2_sensor_lambda

0x7E0->0x7E8 0x114E dpf_soot_mass_measured

0x7E0->0x7E8 0x114F dpf_soot_mass_calculated_or_misfire_variant

0x7E0->0x7E8 0x115E time_since_last_regeneration

0x7E0->0x7E8 0x11BE engine_oil_temperature_2

0x7E0->0x7E8 0x162F dpf_pressure_sensor_raw

0x7E0->0x7E8 0x178C dpf_oil_ash_residue

0x7E0->0x7E8 0x1024 air_mass

0x7E0->0x7E8 0x103C intake_manifold_pressure

0x7E0->0x7E8 0x1044 dpf_surface_temperature

0x7E0->0x7E8 0x1047 engine_torque

0x7E0->0x7E8 0x1057 boost_pressure

0x7E0->0x7E8 0x10C2 egr_duty

## Audi RS e-tron GT 2022 / J1 examples

0x710->0x77A 0x2A07 12v_battery_voltage

0x710->0x77A 0x2A09 12v_battery_current

0x710->0x77A 0x2A0B 12v_battery_temperature

0x710->0x77A 0x2A0C 12v_battery_charge_condition

0x710->0x77A 0x2A53 dcdc_low_voltage_and_current

0x710->0x77A 0x2AB2 hv_battery_capacity

0x710->0x77A 0x2AB5 range_estimate

0x710->0x77A 0x2AB8 hv_battery_energy

0x713->0x77D 0x2B29 steering_angle

0x744->0x7AE 0x1507 charger_rail_energy

0x744->0x7AE 0x15D5 max_charge_power

0x744->0x7AE 0x15D6 charger_efficiency

0x744->0x7AE 0x15E1 charger_power_loss

0x744->0x7AE 0x15E2 charger_coolant_temperature

0x744->0x7AE 0x1DA7 charger_output_voltage

0x746->0x7B0 0x2603 compressor_speed

0x746->0x7B0 0x2609 outside_temperature

0x746->0x7B0 0x2613 cabin_temperature

0x767->0x7D1 0x2431 gps_satellites

0x7E5->0x7ED 0x028C hv_battery_soc

# 7. Positive and negative command support

Negative support is retained with exact vehicle/platform scope. A rejection on a 2022 RS e-tron GT can reduce J1 scan budget, but must not invalidate the same DID on MQB/MEB/MLB.

## Audi Q5 2015

Longitudinal Audi supports dense engine/transmission UDS/OBD; do not assume MQB/MEB body/EV routes.

## Audi Rs Etron Gt 2022

J1 supports gateway, charger, HVAC, BMS and selected brakes/cluster data, while many generic VAG candidates are explicitly rejected.

## Vw Id4

MEB has rich 0x710/0x77A energy-management and 0x744/0x7AE charging data.

# 8. Session, SFD and transport restrictions

Product mode remains in the default diagnostic session. Automatic SecurityAccess, SFD unlock, writes, routines, resets and flashing are denied.

Generic 29-bit enumeration is disabled. FC00/FE00xx command families remain non-executable until their exact raw transport encoding is normalized and physically reproduced.

# 9. Budgets and timeouts

## Known Route Phase

{"max_requests": 80, "rate_requests_per_second": 4, "burst_max": 4, "max_outstanding": 1, "retry_on_timeout": 0}

## Identity Per Ecu

{"max_requests": 7, "rate_requests_per_second": 2, "retry_on_timeout": 1, "retry_backoff_ms": 250}

## Remaining Vag Catalog

{"max_requests": 100, "rate_requests_per_second": 4, "max_outstanding": 1}

## Generic Can11

{"authorized_only_after_transport_detection": true, "from": "0x700", "to": "0x7F6", "exclude": ["0x7DF"], "max_requests": 247, "rate_requests_per_second": 4}

## Global

{"max_requests": 650, "max_wallclock_ms": 180000, "phase_cooldown_ms": 500}

Timeouts: {"presence_first_response_ms": 250, "identity_first_response_ms": 500, "isotp_multiframe_completion_ms": 3000, "response_pending_max_ms": 3000}

# 10. ECU-family hypotheses

## vag_j533_mqb_gateway - gateway

Gateway route/identity reusable only after part/software/chassis family match.

Route: 0x710->0x77A

## vag_gw2020_mqb_evo - gateway

SFD-capable branch; read-only measuring values should remain accessible without unlock.

## vag_meb_gateway_energy - gateway_energy_management

Strong cross-MEB candidate; require ECU fingerprint before sharing decoders.

Route: 0x710->0x77A

## vag_meb_obc - onboard_charger

Route recurs across EV datasets; formulas remain platform/ECU scoped.

Route: 0x744->0x7AE

## audi_j1_bms - hv_battery

J1-specific until compared against another J1 car and exact part/software match.

Route: 0x7E5->0x7ED

## audi_mlb_powertrain - engine_transmission

Good longitudinal Audi starting point; do not import MQB/MEB body routes by default.

Route: 0x7E0/0x7E1

# 11. Key conflicts / non-generalization rules

## vag_one_profile_many_architectures

Finding: VW/Audi spans PQ, MQB, MEB, MLB, J1, PPE and PPC.

Resolution: Platform classification precedes proprietary scanning.

## fc00_transport_unresolved

Finding: Audi/VAG OBDb contains FC00/FE00xx targeted command syntax.

Resolution: Keep non-executable until exact raw transport is normalized and physically captured.

## j1_negative_evidence

Finding: 2022 RS e-tron GT explicitly rejects many VAG make-level MQB/PQ candidates.

Resolution: Store rejection as model/platform-scoped negative evidence; reduce scan budget on J1 without affecting MQB/MEB.

## ppe_ppc_no_routes

Finding: OEM sources confirm PPE/PPC/E3 architectures, but this pass did not establish safe public diagnostic route sets.

Resolution: Terminal platform_known_route_knowledge_incomplete; do not inherit MQB/MLB routes.

## sfd_readonly

Finding: SFD protects advanced changes, while read-only identification/measuring values are expected to remain available.

Resolution: No automatic unlock; continue read-only discovery.

# 12. Physical validation plan

VW Golf Mk7 (vw_mqb): Large parc; core transverse VAG reuse

VW Golf Mk8 (vw_mqb_evo): Modern/SFD branch

VW Polo Mk6 (vw_mqb_a0): High-volume Spain small-car platform

VW ID.4 or ID.3 (vw_meb_gen1): MEB EV route/DID validation

Audi A3 8V/Q3 F3 (audi_mqb): Audi transverse MQB reuse

Audi A4/Q5 longitudinal (audi_mlb): Longitudinal split and negative-evidence validation

Audi e-tron/Q8 e-tron (audi_mlb_evo_ev): First Audi EV architecture

Audi e-tron GT (audi_j1): J1 exact support matrix

Audi Q4 e-tron (audi_meb): Audi MEB cross-brand reuse

Audi Q6/A6 e-tron (audi_ppe): New architecture; route discovery from scratch

# 13. Source ledger

[S00] Current project uds-map.json
URL: local:/mnt/data/uds-map.json
Type: project_knowledge_map | Licence: project_internal | Retrieved: 2026-08-31
Revision: map-v9
Scope: Current VW/Audi VAG profile

[S01] openDBC Volkswagen values.py
URL: https://github.com/commaai/opendbc/blob/master/opendbc/car/volkswagen/values.py
Type: open_source_platform_and_fw_query | Licence: MIT | Retrieved: 2026-08-31
Revision: 9a7851b662dd94df155057ad80c4a00f67b630d8
Scope: VW/Audi platform flags, WMIs, VIN/chassis classification and firmware query

[S02] VAG UDS CAN IDs / ODIS extraction
URL: https://github.com/ConnorHowell/vag-uds-ids
Type: community_odis_extraction | Licence: NOASSERTION | Retrieved: 2026-08-31
Revision: 27b5431ed22a10a41095517b88dc95b3ae212441
Scope: VAG UDS route catalogue

[S03] OBDb Volkswagen make-level signalset
URL: https://github.com/OBDb/Volkswagen/blob/main/signalsets/v3/default.json
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-31
Scope: Volkswagen make-level mixed-platform commands

[S04] OBDb Audi make-level signalset
URL: https://github.com/OBDb/Audi/blob/main/signalsets/v3/default.json
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-31
Scope: Audi make-level mixed-platform commands

[S05] OBDb Volkswagen ID.4 signalset
URL: https://github.com/OBDb/Volkswagen-ID.4/blob/main/signalsets/v3/default.json
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-31
Scope: VW ID.4 / MEB model-scoped data

[S06] OBDb Audi Q5 2015 command-support matrix
URL: https://github.com/OBDb/Audi-Q5/blob/main/tests/test_cases/2015/command_support.yaml
Type: physical_command_support_matrix | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-31
Scope: Audi Q5 2015; MLB/PQ-style longitudinal platform

[S07] OBDb Audi RS e-tron GT 2022 command-support matrix
URL: https://github.com/OBDb/Audi-RS-e-tron/blob/main/tests/test_cases/2022/command_support.yaml
Type: physical_command_support_matrix | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-31
Scope: Audi RS e-tron GT 2022 / J1

[S08] OBDb Audi Q4 e-tron signalset
URL: https://github.com/OBDb/Audi-Q4-e-tron/blob/main/signalsets/v3/default.json
Type: open_diagnostic_database | Licence: CC-BY-SA-4.0 | Retrieved: 2026-08-31
Revision: ad37e0e5bed5520dd705b2eec65834419137edd1
Scope: Audi Q4 e-tron / MEB; current sparse default

[S09] Audi: four electric platforms
URL: https://www.audi-mediacenter.com/system/production/uploaded_files/15615/file/0a6787a9d7a79661577444d90ce2a6692bf4cf84/EN_TD_Insight_e-mobility_Press_Kit_restricted.pdf
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-31
Scope: Audi MLB evo EV, J1, MEB, PPE platform split

[S10] Audi PPE + E3 1.2
URL: https://www.audi-mediacenter.com/en/press-releases/premium-platform-electric-ppe-a-technological-leap-for-the-next-generation-of-fully-electric-premium-mobility-from-audi-16021/download
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-31
Scope: Audi Q6/A6 e-tron PPE electronics architecture

[S11] Audi PPC + E3
URL: https://uploads.audi-mediacenter.com/system/production/uploaded_files/32928/file/fd0c62b07b4f549c21785229054292838e6076fd/20240930_A5_Basisinfo_short_EN.pdf
Type: manufacturer_platform_documentation | Licence: reference_only | Retrieved: 2026-08-31
Scope: New Audi A5/PPC platform

[S12] Ross-Tech SFD Wiki
URL: https://wiki.ross-tech.com/wiki/index.php/SFD
Type: professional_diagnostic_documentation | Licence: reference_only | Retrieved: 2026-08-31
Scope: VAG SFD behavior

# 14. Import guidance

Import as an additive research overlay. Existing vehicle/family-confirmed project facts win. Platform-specific negative evidence remains scoped. PPE/PPC remain platform-known/route-knowledge-incomplete until safe diagnostic captures are obtained.